Battery Controller Charging and Discharging

Voltage source Parameters:

- Amplitude: 48V

- Measurement: voltage

Constant Parameters:

- constant value: 0

- enable the vector parameter as 1-D

- Sample time: inf

IGBT Parameters:

- Internal resistance:0.001 ohm

- initial state:0 (open)

- Snubber resistance : 1e^5 ohm

- Snubber capacitances: inf F

Load resistance Parameter:

- Resistance: 6 ohm

- Measurement: Branch voltage and current

Bidirectional DC-DC Converter:

RC parameters:

- Branch type: RC

- Resistance: 0.003 ohm

- Capacitance: 1000e^-6 F

- Measurement : branch voltage and current

IGBT parameter (S1):

- Internal resistance:0.004 ohm

- Snubber resistance : 1e^5 ohm

- Snubber capacitances: inf F

IGBT parameter (S2):

- Internal resistance:0.004 ohm

- Snubber resistance : 1e^5 ohm

- Snubber capacitances: inf F

RL parameters:

- Branch type: RL

- Resistance: 0.05 ohm

- Inductance: 5.76e^-4 F

- Inductor initial current: 0 A

- Measurement : branch voltage and current

Battery Parameter:

- Type: Lithium Ion

- Nominal voltage: 24 V

- Rated capacity: 50 Ah

- Initial state-of-charge: 45%

- Battery response time: 1 s

Detect mode charging or discharging:

Switch Parameter:

- criteria for passing first input: u2> threshold

- threshold: 0

- enable the zero crossing detection

Generation for ref current of battery charging:

Down sample Parameter:

- factor source: Dialog parameter

- Down sample factor: Fs/Fsw

- Sample offset: 0

- initial condition: 0

- Rate option: Allow multirate processing

PI controller:

- Form: Parallel

- Time domain: Discrete-time

- source: internal

- Proportional: 50

- Integral: 1000

- enable zero crossing detection

Generation for ref current of battery discharging:

Down sample Parameter:

- factor source: Dialog parameter

- Down sample factor: Fs/Fsw

- Sample offset: 0

- initial condition: 0

- Rate option: Allow multirate processing

PI controller:

- Form: Parallel

- Time domain: Discrete-time

- source: internal

- Proportional: 0.5

- Integral: 50

- enable zero crossing detection

Current control of battery in charging and discharging:

PI controller:

- Form: Parallel

- Time domain: Discrete-time

- source: internal

- Proportional: 0.05

- Integral: 10

- enable zero crossing detection

Sawtooth Parameter:

- Time Values: [0 0.0001]

- output values: [0 1]